RADIOGRAPHY TEST REPORT

| Customer Name & Address * : |  | M/s. ESAB INDIA LTD. Ambattur Industrial Estate, Chennai – 600 058. |  |  |  |  |  | URL No : TC16435261000000877F |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Principal Customer * : - - - |  |  |  |  |  |  |  | Report No : JIA / RT- 1431 |  |  |  |  |
| Work Order No & Date * : - - - |  |  |  |  |  |  |  | Issue Date : 01.08.2026 |  |  |  |  |
| Part Name * : Welded Test Plate |  |  |  |  |  |  |  | Date of Examination : 31.07.2026 |  |  |  |  |
| Part No : 932 |  |  |  |  |  |  |  | DC No : 267 |  |  |  |  |
| Heat No * : N.A. |  |  |  |  |  |  |  | Item Receipt Date : 31.07.2026 |  |  |  |  |
| Drawing No.* : N.A. |  |  |  |  |  |  |  | Test Location : Jai Inspection Agencies LLP |  |  |  |  |
| Material * : IS2062 |  |  |  |  |  |  |  | Source : X-Ray / Ir.-192 / Co-60 |  |  |  |  |
| Size & Thickness* : 330mm x 300mm x 12mmThick. |  |  |  |  |  |  |  | Source Strength : 21.00Ci. |  |  |  |  |
| Area Tested * : Butt Welded Area |  |  |  |  |  |  |  | Exposure Time / KV &Ma : 5 Minutes |  |  |  |  |
| Lead Screens : 0.15mm ( Front & Back ) |  |  |  |  |  |  |  | Source Size / Focal Spot : 2.4mm x 2.7mm |  |  |  |  |
| Exposure Technique : S W S I |  |  |  |  |  |  |  | Film Class & Brand : Agfa D7 / Class II |  |  |  |  |
| Test Method * : ASTM E 1032 |  |  |  |  |  |  |  | Developing Temp / Time : 20°C / 5 Minutes |  |  |  |  |
| Acceptance Std.* : ASME SEC II Part C 2025 SFA 5.1 Grade - 1 |  |  |  |  |  |  |  | Penetrameter : ASTM : 20 1B (Hole & Wire ) |  |  |  |  |
| Test Performed by : M. Samson (Radiographer) |  |  |  |  |  |  |  | Test Carried in Presence of : --- |  |  |  |  |
| Sr. No | Description |  | Thick ness |  | Segment | S.F.D | Density | Sensitivity |  | Film Size | Observations |  |
| 1 | Brand : OK 55.00L B. No. RMA3015932 Size: 2.5 x 350nm ID : 932 |  | 20+2mm. |  | A – B | 20” | 2.91 | 2 – 2T & 2% |  | 4 x 16” | N S D - Non Rejectable |  |
| Remarks: - - - |  |  |  |  |  |  |  |  |  |  |  |  |
| ABBREVIATION : N S D - NO SIGNIFICANT DEFECT |  |  |  |  |  |  |  |  |  |  |  |  |
| Evaluated By |  |  |  | Reviewed & Authorized By |  |  |  |  | for Client |  |  | for Inspecting Officer |
| for JAI INSPECTION AGENCIES LLP C. ATHILINGAM NDT LEVEL - II |  |  |  | For JAI INSPECTION AGENCIES LLP E. VIOLA AUTHORIZED SIGNATORY |  |  |  |  |  |  |  |  |

Note : 1.  Observation confirms to the above acceptance standard as confirmed by customer

2.  “ * ” Denotes details provided by customer

3.  Results are related to Test item only.  Any manual corrections will be invalid.  The Test report shall not be reproduced    without the written consent from M/s. Jai Inspection Agencies LLP

Page1 of 1	                                                                                   ****      End of Report      ****

RADIOGRAPHY TEST REPORT

| Customer Name & Address * : |  | M/s Perfect Material Testing Private Limited Singaravelan Nagar, Maduravoyal, Chennai – 95 |  |  |  |  |  | URL No : TC16435261000000878F |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Principal Customer * : M/s. SVJ Laser Tech, Vellanur, Avadi-600062. |  |  |  |  |  |  |  | Report No : JIA / RT- 1432 |  |  |  |  |
| Work Order No & Date * : - - - |  |  |  |  |  |  |  | Issue Date : 01.08.2026 |  |  |  |  |
| Part Name * : MS PLATE |  |  |  |  |  |  |  | Date of Examination : 01.08.2026 |  |  |  |  |
| Part No * : N.A. |  |  |  |  |  |  |  | DC No : 1873 |  |  |  |  |
| Heat No * : N.A. |  |  |  |  |  |  |  | Item Receipt Date : 31.07.2026 |  |  |  |  |
| Drawing No.* : - - - |  |  |  |  |  |  |  | Test Location : Jai Inspection Agencies LLP |  |  |  |  |
| Material * : IS2062 E250A |  |  |  |  |  |  |  | Source : X - Ray / IR-192 / Co-60 |  |  |  |  |
| Size & Thickness* : 2mm. |  |  |  |  |  |  |  | Source Strength : -- |  |  |  |  |
| Area Tested * : 100% X - Ray |  |  |  |  |  |  |  | Expos. Time / KV & Ma : 3minutes / 120 & 3 |  |  |  |  |
| Lead Screens : 0.15mm ( Front & Back ) |  |  |  |  |  |  |  | Source Size / Focal Spot: 1.5 x 1.5mm. |  |  |  |  |
| Exposure Technique : S W S I |  |  |  |  |  |  |  | Film Class & Brand : Agfa D7 |  |  |  |  |
| Test Method * : ASME SEC.V, Edition 2025 |  |  |  |  |  |  |  | Penetrameter : DIN : 1 A |  |  |  |  |
| Acceptance Std. * : ASME SEC.IX, Edition 2025 |  |  |  |  |  |  |  | Developing Temp / Time : 20°C / 5 Minutes |  |  |  |  |
| Test Performed by : M. Samson (Radiographer) |  |  |  |  |  |  |  | Test Carried in Presence of : --- |  |  |  |  |
| Sr. No | Description |  | Thick ness |  | Segment | S.F.D | Density | Sensitivity |  | Film Size | Observations |  |
| 1 | 2mm - PLATE |  | 2mm |  | A – B | 24” | 1.8 – 2.6 | 2% |  | 4 x 6” | N S D – Non Rejectable |  |
| 2 | 2mm - PLATE |  | 2mm |  | C - D | 24” | 1.8 – 2.6 | 2% |  | 4 x 6” | N S D – Non Rejectable |  |
| Remarks: - - - |  |  |  |  |  |  |  |  |  |  |  |  |
| ABBREVIATION : N S D – NO SIGNIFICANT DEFECT |  |  |  |  |  |  |  |  |  |  |  |  |
| Evaluated By |  |  |  | Reviewed & Authorized By |  |  |  |  | for Client |  |  | for Inspecting Officer |
| for JAI INSPECTION AGENCIES LLP C. ATHILINGAM NDT LEVEL II |  |  |  | For JAI INSPECTION AGENCIES LLP E. VIOLA AUTHORIZED SIGNATORY |  |  |  |  |  |  |  |  |

Note : 1.  Observation confirms to the above acceptance standard as confirmed by customer

2.  “ * ” Denotes details provided by customer

3.  Results are related to Test item only.  Any manual corrections will be invalid.  The Test report shall not be reproduced    without the written consent from M/s. Jai Inspection Agencies LLP

Page1 of 1	                                                             ****      End of Report      ****